CMPG325-2026-126

INDIVIDUAL SEMESTER PROJECT

MILESTONE 1: CLIENT DESIGN REVIEW

| Project ID | CMPG325-2026-126 |
| --- | --- |
| Student | SELOWA, S |
| Student Number | 45192472 |
| Client ID | CLI-126 |
| Organisation | Moribo Live Music Venue (Kimberley) |
| Industry | Entertainment |
| Date | 28 August 2026 |

TABLE OF CONTENTS

- Client Requirements

- Physical Topology

- Logical Topology

- IP Addressing Plan

- Initial GitHub Repository

- Design Decisions Summary

- Next Steps

1. CLIENT REQUIREMENTS

1.1 Client Overview

| Attribute | Details |
| --- | --- |
| Client ID | CLI-126 |
| Organisation | Moribo Live Music Venue |
| Location | Kimberley, South Africa |
| Industry | Entertainment |
| Assigned Addressing Block | 172.30.80.0/23 |
| Assigned Networking Challenge | Inter-VLAN Routing (L3 switch / SVI) |
| Design Constraint | Remote management of network devices for off-site IT contracto |
| Change Request | CR10: Four additional point-of-sale terminals deployed |

1.2 Business Context

Moribo Live Music Venue is an entertainment establishment located in Kimberley. The venue hosts live music events, concerts, and performances on a regular basis. As a busy entertainment venue, it requires a robust and reliable computer network to support its daily operations, including:

- Ticketing and admissions – Processing ticket sales at the entrance

- Merchandise sales – Selling band merchandise and venue memorabilia

- Concessions and bar operations – Processing food and beverage payments

- Administrative functions – Staff workstations for management and operations

- Guest services – Providing Wi-Fi access to patrons

- IT management – Remote monitoring and management by external IT contractor

1.3 Functional Requirements

Based on the client brief, the network must satisfy the following functional requirements:

| Requirement ID | Description |
| --- | --- |
| R1 | Provide appropriate connectivity and network services for the venue |
| R2 | Accommodate Inter-VLAN Routing using a Layer 3 switch with SVIs |
| R3 | Enable remote management of network devices for off-site IT contractor |
| R4 | Integrate four additional POS terminals (Change Request CR10) |
| R5 | Support successful data exchange between appropriate nodes |
| R6 | Be testable and verifiable in Cisco Packet Tracer |

1.4 Technical Requirements

| Category | Specification |
| --- | --- |
| Simulation Tool | Cisco Packet Tracer |
| Addressing Block | 172.30.80.0/23 |
| Routing Method | Inter-VLAN Routing (L3 switch / SVI) |
| Management Protocol | SSH (Secure Shell) for remote access |
| VLAN Count | Minimum of 4 VLANs |
| Difficulty Level | Intermediate |

1.5 Design Constraints

The following constraints must be addressed in the network design:

- Remote Management Constraint: The off-site IT contractor must be able to securely access and manage network devices from an external location. This requires SSH configuration on all network infrastructure devices.

- Addressing Constraint: All IP addressing must be derived from the assigned block 172.30.80.0/23 using appropriate subnetting.

- Change Request CR10: The network must accommodate four additional POS terminals beyond the existing two, bringing the total to six POS terminals.

1.6 Change Request CR10 Analysis

| Aspect | Details |
| --- | --- |
| Change Request ID | CR10 |
| Description | Four additional point-of-sale terminals deployed |
| Impact | Network expansion – POS device count increases from 2 to 6 |
| Implementation | Add new POS devices to existing POS VLAN and subnet |
| Addressing Impact | None – POS subnet has sufficient capacity |
| Performance Impact | Minimal – six POS terminals generate low network traffic |

1.7 Assumptions

The following assumptions were made during the design process:

The venue operates from a single physical location in Kimberley.

- All end devices (staff PCs, POS terminals) are connected via wired Ethernet.

- Guest Wi-Fi is provided via a wireless access point for patrons.

- Network devices are managed via SSH with local authentication.

- DHCP is used for dynamic IP assignment to end devices.

- The venue does not require a dedicated server for file sharing or database services.

- The off-site IT contractor has a reliable internet connection for remote management.

2. PHYSICAL TOPOLOGY

2.1 Topology Overview

The physical topology adopts a hierarchical star-based design with a central distribution layer (Layer 3 switch) and multiple access switches (Layer 2 switches) connecting end devices. This design provides:

- Scalability: Easy addition of new devices and switches

- Manageability: Centralised control and monitoring

- Performance: Dedicated links between access and distribution layers

- Simplicity: Clear separation of core, distribution, and access layers

2.2 Topology Diagram

2.3 Device Inventory

| Device Type | Model | Quantity | Role | VLAN Assignment |
| --- | --- | --- | --- | --- |
| Layer 3 Switch | Cisco 3560-24PS | 1 | Distribution/Core routing | VLAN 10 (Management) |
| Layer 2 Switch 1 | Cisco 2960-24TT | 1 | Access layer | VLAN 10 (Management) |
| Layer 2 Switch 2 | Cisco 2960-24TT | 1 | Access layer | VLAN 10 (Management) |
| Wireless Router | WRT300N | 1 | Guest Wi-Fi access | VLAN 40 (Guest) |
| Staff PCs | PC-PT | 4 | Administrative workstations | VLAN 20 (Staff) |
| POS Terminals | PC-PT | 6 | Point-of-Sale systems | VLAN 30 (POS) |
| Management PC | PC-PT | 1 | Network administration | VLAN 10 (Management) |

2.4 Physical Connections

| Connection | Cable Type | Source Port | Destination Port |
| --- | --- | --- | --- |
| L3-SW ↔ L2-SW1 | Copper Cross-over | Gig0/1 | Gig0/1 |
| L3-SW ↔ L2-SW2 | Copper Cross-over | Gig0/2 | Gig0/1 |
| L3-SW ↔ WRT300N | Copper Straight-through | Gig0/3 | Internet Port |
| L3-SW ↔ Management PC | Copper Straight-through | Fa0/1 | FastEthernet0 |
| L2-SW1 ↔ Staff PC1 | Copper Straight-through | Fa0/1 | FastEthernet0 |
| L2-SW1 ↔ Staff PC2 | Copper Straight-through | Fa0/2 | FastEthernet0 |
| L2-SW1 ↔ POS Terminal 1 | Copper Straight-through | Fa0/3 | FastEthernet0 |
| L2-SW1 ↔ POS Terminal 2 | Copper Straight-through | Fa0/4 | FastEthernet0 |
| L2-SW1 ↔ POS Terminal 3 (CR10) | Copper Straight-through | Fa0/5 | FastEthernet0 |
| L2-SW1 ↔ POS Terminal 4 (CR10) | Copper Straight-through | Fa0/6 | FastEthernet0 |
| L2-SW2 ↔ Staff PC3 | Copper Straight-through | Fa0/1 | FastEthernet0 |
| L2-SW2 ↔ Staff PC4 | Copper Straight-through | Fa0/2 | FastEthernet0 |
| L2-SW2 ↔ POS Terminal 5 (CR10) | Copper Straight-through | Fa0/3 | FastEthernet0 |
| L2-SW2 ↔ POS Terminal 6 (CR10) | Copper Straight-through | Fa0/4 | FastEthernet0 |

3. LOGICAL TOPOLOGY

3.1 VLAN Design

The network will be segmented into the following VLANs based on functional requirements and industry best practices:

| VLAN ID | VLAN Name | Purpose | Subnet | Gateway (SVI) |
| --- | --- | --- | --- | --- |
| 10 | Management | Network device management (SSH access) | 172.30.80.0/26 | 172.30.80.1 |
| 20 | Staff | Administrative and office staff workstations | 172.30.80.64/26 | 172.30.80.65 |
| 30 | POS | Point-of-Sale terminals (includes CR10 expansion) | 172.30.80.128/26 | 172.30.80.129 |
| 40 | Guest | Public Wi-Fi for venue patrons | 172.30.80.192/27 | 172.30.80.193 |
| 99 | Native | Untagged traffic on trunk links | N/A | N/A |

3.2 VLAN Design Rationale

| VLAN | Rationale |
| --- | --- |
| VLAN 10 (Management) | Isolates management interfaces for security. IT contractor can access all devices via SSH on this VLAN. Aligns with best practices for secure device management. |
| VLAN 20 (Staff) | Separates administrative traffic from operational systems. Prevents staff devices from directly accessing POS or management networks. |
| VLAN 30 (POS) | Critical security isolation for payment card environment. Protects POS traffic from general network traffic. Provides dedicated bandwidth for payment processing. |
| VLAN 40 (Guest) | Isolates public Wi-Fi traffic from internal business networks. Prevents guest devices from accessing sensitive systems. Provides internet-only access. |

3.3 Logical Topology Diagram

3.4 Inter-VLAN Routing Design

Inter-VLAN routing will be implemented using Switch Virtual Interfaces (SVIs) on the Layer 3 switch (Cisco 3560). This approach is chosen because:

| Advantage | Description |
| --- | --- |
| Performance | Hardware-based routing at near-wire speed |
| Efficiency | Eliminates need for external router |
| Scalability | SVIs can be easily added for new VLANs |
| Management | Centralised routing configuration |
| Cost-effective | Utilises existing switch hardware |

SVI Configuration Overview:

interface vlan 10

ip address 172.30.80.1 255.255.255.192

no shutdown

interface vlan 20

ip address 172.30.80.65 255.255.255.192

no shutdown

interface vlan 30

ip address 172.30.80.129 255.255.255.192

no shutdown

interface vlan 40

ip address 172.30.80.193 255.255.255.224

no shutdown

3.5 Remote Management Design

To satisfy the remote management constraint, the following design decisions were made:

| Feature | Implementation |
| --- | --- |
| Secure Access | SSH v2 configured on all switches |
| Management VLAN | All management interfaces on VLAN 10 |
| VTY Lines | Configured for SSH only (Telnet disabled) |
| Authentication | Local user database with encrypted passwords |
| Domain Name | moribo.local for SSH key generation |

SSH Configuration Overview:

username admin secret admin123

line vty 0 4

transport input ssh

login local

ip ssh version 2

ip domain-name moribo.local

crypto key generate rsa modulus 1024

6. DESIGN DECISIONS SUMMARY

| Decision | Rationale |
| --- | --- |
| Layer 3 switch (3560) for routing | Provides high-performance inter-VLAN routing via SVIs; eliminates need for separate router |
| VLAN segmentation (4 VLANs) | Isolates management, staff, POS, and guest traffic for security and performance |
| /26 subnets for core VLANs | Provides 62 usable addresses per VLAN, accommodating growth |
| Dedicated POS VLAN | Essential security practice for payment card environments |
| SSH-only remote management | Secure alternative to Telnet; satisfies off-site IT contractor constraint |
| DHCP for end devices | Simplifies administration and reduces configuration errors |
| Static addressing for infrastructure | Ensures consistent, predictable management access |
| Hierarchical topology | Provides scalability, manageability, and performance |
| Trunk links between switches | Allows multiple VLANs to traverse a single physical connection |
| WRT300N for guest Wi-Fi | Provides isolated wireless access for patrons |

6.1 Key Design Principles Applied

- Security First – Management and POS traffic are isolated in dedicated VLANs

- Scalability – Subnet sizes accommodate future growth

- Simplicity – Design is easy to implement and maintain

- Best Practices – Follows Cisco and industry networking standards

- Remote Management – SSH configuration for off-site access

6.2 Change Request CR10 Integration

| Aspect | Implementation |
| --- | --- |
| Impact Assessment | POS VLAN has 62 addresses, currently uses 2, adding 4 is minimal impact |
| Addressing | No subnet changes required – new terminals use existing DHCP pool |
| Physical Connectivity | Additional POS terminals connected to available switch ports |
| Configuration | Set switchports to access VLAN 30 for new POS terminals |
| Testing | Verify connectivity and DHCP lease assignment |

7. NEXT STEPS (MILESTONE 2 PREPARATION)

The following tasks will be completed for Milestone 2 (due 2 October 2026):

7.1 Implementation Tasks

| Task | Description | Status |
| --- | --- | --- |
| 1. Build Physical Topology | Create network in Cisco Packet Tracer | Pending |
| 2. Configure VLANs | Create VLANs 10, 20, 30, 40 on all switches | Pending |
| 3. Configure Trunk Links | Set up 802.1Q trunking between switches | Pending |
| 4. Configure SVIs | Set up SVIs on Layer 3 switch for inter-VLAN routing | Pending |
| 5. Enable ip routing | Enable routing on the Layer 3 switch | Pending |
| 6. Configure DHCP | Set up DHCP pools on L3 switch | Pending |
| 7. Configure SSH | Enable secure remote management | Pending |
| 8. Connect End Devices | Add PCs, POS terminals, and WRT300N | Pending |
| 9. Test Connectivity | Verify all VLANs can communicate | Pending |
| 10. Verify CR10 | Confirm four new POS terminals work | Pending |

7.2 Testing Plan

| Test | Method | Expected Result |
| --- | --- | --- |
| Intra-VLAN Ping | Ping within same VLAN | Successful |
| Inter-VLAN Ping | Ping between different VLANs | Successful (routed) |
| SSH Remote Access | SSH from management PC to L3-SW | Successful login |
| DHCP Assignment | Check IP addresses on end devices | Correct subnet IPs |
| POS Connectivity | Ping from POS to gateway | Successful |
| CR10 Verification | All 6 POS terminals online | Working |

7.3 Documentation to Complete

- Packet Tracer .pkt file

- Device configuration files

- Connectivity test results with screenshots

- SVI verification results

- SSH access verification

- CR10 implementation evidence

8. REFERENCES

- Cisco Systems. (2023). Inter-VLAN Routing Configuration Guide. Cisco Documentation. Retrieved from https://www.cisco.com/

- Cisco Systems. (2023). Catalyst 3560 Switch Software Configuration Guide. Cisco Documentation.

- Cisco Systems. (2023). Catalyst 2960 Switch Software Configuration Guide. Cisco Documentation.

- Cisco Business. (2024). VLAN Best Practices and Security Tips. Cisco Business Blog.

- CMPG325 Project Handbook. (2026). North-West University.

- Odom, W. (2020). CCNA 200-301 Official Cert Guide Library. Cisco Press.

- Graziani, R., & Johnson, A. (2018). Routing and Switching Essentials Companion Guide. Cisco Networking Academy.